MPPT Solar Charge Controller

Communication Protocol

Series port

Format：8 data bits 1 stop bits no check bits

Baud rate：9600bps

Communication format：

0x55 0xAA 0x01 0x00 0x00 0x00 0x00   【********】 0x01 0x00

Frame header: 55  AA

address : 01

control code: 00

function code: 00

data length: 00  00

data:【********】

sum check: 01  00

For example：

send：0x55 0xAA 0x01 0x00 0x00 0x00 0x00 0x01 0x00

return：0x55 0xAA 0x01  0x00 0x00 0xxx 0xxx

Read:

Control code: 00

1.Function code: 00

| Measuring Channels | Unit | Description | Length |  |
| --- | --- | --- | --- | --- |
| datalen |  | Data length | 2 Bytes |  |
| PvVolt | 0. 1V | Pv voltage | 2 Bytes |  |
| BusVolt | 0.1V | Bus voltage | 2 Bytes | Not applicable |
| BuckVolt | 0.1V | Buck voltage | 2 Bytes | Not applicable |
| Battery voltage | 0.1V | Output voltage | 2 Bytes |  |
| Charging current | 0.01A | Out current | 2 Bytes |  |
| Charging power | 0.1W | Out power | 2 Bytes |  |
| Battery temperature | ℃ | Battery temperature | 2 Bytes |  |
| ------ | ℃ | Buck1 temperature | 2 Bytes | Not applicable |
| Inside temperature | ℃ | Buck2 temperature | 2 Bytes | Not applicable |
| ------ | ℃ | Environment Temperature | 2 Bytes | Not applicable |
| Operating mode |  | POWER_ON_MODE 00 00 START_CHECK_MODE 00 01 NORMAL_RUN_MODE 00 02 FAULT_MODE 00 03 UPDATE_MODE 00 04 SETPAR_MODE 00 05 | 2 Bytes |  |
| ------- | 00 | 00 | 1Byte |  |
| day power output | wh | 00 00 00 00 | 4Byte |  |
| Total power output | wh | 00 00 00 00 | 4 Byte |  |
| Charging mode |  | 00 no charge 01 cc charge 02 cv charge 03 float charge 04 soft start mode | 1 |  |
| Reserve mode |  | 00 00 | 2 |  |
| Error information |  | Error message（see lable2） | 1 Bytes |  |
| Resever mode |  | 00 00 00 00 | 3 Byte |  |
| Sum check |  | xx xx |  |  |

2. Function code: 01

| Measuring Channels | Unit | Description | Length |  |
| --- | --- | --- | --- | --- |
| datalen |  | Data length | 2 Bytes |  |
| PVVoltOffSet | 0.1V | Pv voltage adjustment | 2 Bytes |  |
| PVVoltRatio |  | PV voltage ratio | 2 Bytes | Not applicable |
| ----- | 0.1V | Bus Voltage adjustment | 2 Bytes | Not applicable |
| ----- |  | Bus voltage ratio | 2 Bytes | Not applicable |
| ----- | 0.1V | Buck Voltage adjustment | 2 Bytes | Not applicable |
| ----- |  | Buck voltage ratio | 2 Bytes | Not applicable |
| OutVoltOffset | 0.1V | Out Voltage adjustment | 2 Bytes |  |
| OutVoltRatio |  | Out voltage ratio | 2 Bytes |  |
| OutCurOffset | 0.01A | Out current adjustment | 2 Bytes |  |
| OutCurRatio |  | Out current ratio | 4 Bytes |  |
| SerialNum | ASCII | Serial number | 16Bytes |  |
| MachineAdderess |  | Machine address | 1 Bytes |  |
| BatteryType |  | Battery type | 1 Bytes |  |
| MachineModel | ASCII | Machine model | 32Byte |  |
| Firmware | ASCII |  | 4Byte |  |
| Max Charge Current | 0.01A |  | 2 |  |
| Buck Charge Voltage | 0.1V |  | 2 |  |
| Float Charge Voltage | 0.1V |  | 2 |  |

3.Function code: 03

| Measuring Channels | Unit | Description | Length |  |
| --- | --- | --- | --- | --- |
| Date Len |  |  | 2Byte |  |
| Machine Type |  | (01:charge 02: invert 03:combo) | 1Byte |  |
| SerialNum |  | Serial number | 16Bytes |  |
| BatteryType |  | 01: Vented 02:Sealed 03:Gel 04:NiCd | 1 Bytes |  |
| Machine Mode |  | ASCII | 32BYTE |  |
| MAIN Fireware |  | ASCII | 4Byte |  |
| SLAVE Fireware |  | ASCII | 4Byte |  |
| First power Rcode date |  | Year month day hour minute second | 6Byte |  |

4.Function:05: SENT：0x55 0xaa 0x01 00 05 00 01 Year month Sum1 Sum2  (read monthy output power)

| Measuring Channels | Unit | Description | Length |  |
| --- | --- | --- | --- | --- |
| Date Len |  |  | 2 |  |
| Year |  |  | 1Byte |  |
| month |  |  | 1Byte |  |
| RETURN DATA EXAMPLE: 0x55 0xaa 0x01 00 05 00 04 ** ** ** ** Sum1 Sum2 |  |  |  |  |
| Power |  |  | 4Byte |  |

5.Function 06: sent：0x55 0xaa 0x01 00 06 00 01 month day Sum1 Sum2  (read monthy output power)

| Measuring Channels | Unit | Description | Length |  |
| --- | --- | --- | --- | --- |
| Date Len |  |  | 2 |  |
| Year |  |  | 1Byte |  |
| month |  |  | 1Byte |  |
| RETURN DATA EXAMPLE: 0x55 0xaa 0x01 00 06 00 02 ** ** Sum1 Sum2 |  |  |  |  |
| Power |  |  | 2Byte |  |

6.Function 07: sent：0x55 0xaa 0x01 00 07 00 01 day hour Sum1 Sum2  (read monthy output power)

| Measuring Channels | Unit | Description | Length |  |
| --- | --- | --- | --- | --- |
| Date Len |  |  | 2 |  |
| Year |  |  | 1Byte |  |
| month |  |  | 1Byte |  |
| RETURN DATA EXAMPLE: 0x55 0xaa 0x01 00 07 00 02 ** ** Sum1 Sum2 |  |  |  |  |
| Power |  |  | 2Byte |  |

7.Function code:08

| Measuring Channels | Unit | Description | Length |  |
| --- | --- | --- | --- | --- |
| Date Len |  |  | 6 |  |
| Year |  |  | 1Byte |  |
| month |  |  | 1Byte |  |
| day |  |  | 1 Bytes |  |
| hour |  |  | 1Byte |  |
| minute |  |  | 1Byte |  |
| second |  |  | 1Byte |  |

Write:

Control code: 01

| Control code | Function code | Name | Unit | Length |
| --- | --- | --- | --- | --- |
| 01 | 00 | PVVoltOffSet | 0.1V |  |
|  | 01 | PVVoltRatio |  |  |
|  | 02 | BUSVoltOffSet | 0.1V | Not applicable |
|  | 03 | BUSVoltRatio |  | Not applicable |
|  | 04 | BuckVoltOffSet | 0.1V | Not applicable |
|  | 05 | BuckVoltRatio |  | Not applicable |
|  | 06 | OutVoltOffSet | 0.1V |  |
|  | 07 | OutVoltRatio |  |  |
|  | 08 | OutCurOffSet | 0.01A |  |
|  | 09 | OutCurRatio |  |  |
|  | 0A | SerialNum |  |  |
|  | 0B | MachineAdderess | 1byte |  |
|  | 0C | BatteryType 00: User define 01: Vented 02:Sealed 03:Gel 04:NiCd | 1 byte |  |
|  | 0D | Total power(Clear Total power) | 0 byte |  |
|  | 0E | Max Charge Current | 0.01A | 2 |
|  | 0F | Buck charge voltage | 0.1V | 2 |
|  | 10 | Float charge voltage | 0.1V | 2 |
|  | 11 | Year/month/day/hour/minute /second |  | 6 |
|  | 2D | Load off battery voltage |  | 1 |
|  | 2E | Load ON Mode |  | 0 |
|  | 2F | Load OFF Mode |  | 0 |

# Control Code: 02 ‘Execute’

| Control code | Function code | description |
| --- | --- | --- |
| 0x02 | 00 | Initial param |
|  | 09 | Set emergency stop fault |
|  | 0A | Clear emergency stop fault |

Control Code: 61 ‘Read’ (Only for TCP)

| Measuring Channels | Unit | Description | Length |  |
| --- | --- | --- | --- | --- |
| Datalen |  | Data length | 2 Bytes | 5 6 |
| Local IP Address |  |  | 4 |  |
| Mask IP |  |  | 4 |  |
| Gate IP |  |  | 4 |  |
| Local TCP Port |  |  | 2 |  |
| Local UDP Port |  |  | 2 |  |
| TCP Severice IP |  |  | 4 |  |
| TCP Severice Port |  |  | 2 |  |
| UDP Severice IP |  |  | 4 |  |
| UDP Severice Port |  |  | 2 |  |
| Mac Address |  |  | 6 |  |

Control Code: 62 ‘Write’(Only for TCP)

| Control Code | Function Code | Description | Length |  |
| --- | --- | --- | --- | --- |
| 0x62 | 1 | Local IP Address | 4 |  |
|  | 2 | Mask IP | 4 |  |
|  | 3 | Gate IP | 4 |  |
|  | 4 | Local TCP Port | 2 |  |
|  | 5 | TCP Severice IP | 4 |  |
|  | 6 | TCP Severice Port | 2 |  |
|  | 7 | UDP Severice IP | 4 |  |
|  | 8 | UDP Severice Port | 2 |  |
|  | 9 | Mac Address | 6 |  |
|  | 10 | Local UDP Port | 2 |  |

[*label2]

|  | Error message | Description |  |
| --- | --- | --- | --- |
| 0 | InRelay | Input relay failure |  |
| 1 | InUvp | Input voltage low |  |
| 2 | InOvp | Input voltage high |  |
| 3 | Buck1Ocp | Buck1 current high | Not applicable |
| 4 | Buck2Ocp | Buck2 current high | Not applicable |
| 5 | OutUvp | Output voltage low |  |
| 6 | OutOvp | Output voltage high |  |
| 7 | OutRelay | Output relay failure | Not applicable |
| 8 | OutShort | Output circuit short |  |
| 9 | PolarError | Polar connect error |  |
| 10 | BatVoltError | Battery voltage error |  |
| 11 | Buck1Fault | Buck1 circuit fault | Not applicable |
| 12 | D1MosfetFault | D1 mosfet failure |  |
| 13 | Buck2Fault | Buck2 circuit fault |  |
| 14 | D2MosfetFault | D2 mosfet failure |  |
| 15 | BatOverTemp | Battery temperature high |  |
| 16 | EnirOverTemp | Environment temperature high | Not applicable |
| 17 | BuckOverTemp | Buck temperature high | Not applicable |
| 18 | Fan1Fault | Fan1 failure | Not applicable |
| 19 | Fan2Fault | Fan2 failure | Not applicable |
| 20 | PvVoltSensorFault | Pv voltage senor failure |  |
| 21 | BusVoltSensorFault | BUS voltage senor failure | Not applicable |
| 22 | BuckVoltSensorFault | Buck voltage senor failure | Not applicable |
| 23 | OutVoltSensorFault | Output voltage senor failure |  |
| 24 | PvCurSensorFault | Pv current senor failure |  |
| 25 | OutCurSensorFault | Output current senor failure |  |
| 26 | BatOffFault | Battery turn off failure |  |
| 27 | EmergencyStopFault | Emergency stop failure |  |
| 29 | BuckOvpFault | Buck voltage high | Not applicable |